(IS TR)

TOGGLE RELAY

The toggle relay is a simple low cost module ideal for use with radio controlled transmitter
/receivers to operate gates, doors, barriers, CCTV cameras and alarm systems. It has been designed for 12 or 24 Volt DC operation and has a double pole relay rated at 8 amps 240 V AC complete with LED indication.

Operation:

The relay is activated by a trigger which can be a positive or negative voltage applied or

removed from the ‘T’ terminal. This gives a toggled on-off action.

Options:

| Link In | Trigger |  | Relay |
| --- | --- | --- | --- |
| A | Applied |  | Not latched |
| B | Removed |  | Not latched |
| D | Removed |  | Latched |
| D | Applied |  | Latched |

| Supply Voltage | 12V DC | 24V DC |
| --- | --- | --- |
| Quiescent Current | 8 mA | 23 mA |
| Current when active | 38 mA | 68 mA |
| Double Pole Relay rated at | 8 Amps at 240 V AC minimum 10 mA |  |
| Dimensions | 57mm (W) 82mm (L) 20mm (H) |  |
| Packing Weight | 50g |  |

We recommend IS Sticky feet size B, 
self-adhesive mounting pillars to ensure
a neat fixing